ADC Lab Logbook

Part 1  |  Section 1: Measuring Impedance

Section 1: Measuring Impedance

Theoretical Background

The impedance of a component is defined by Ohm's law as the ratio of the voltage across it to the current through it. Since an oscilloscope measures voltage rather than current, a known series resistor R is introduced into the circuit. By measuring the voltage across R (denoted Vᵆ) and the voltage across the unknown component (denoted Vᴻ), the impedance can be determined using the potential divider relationship:

Z = R · (Vᴻ / Vᵆ)

To avoid a short circuit caused by the shared ground connection between the signal generator and oscilloscope, the voltage across R is obtained indirectly using the oscilloscope’s MATH subtraction function: Vᵆ = CH1 − CH2, where CH1 measures the total input voltage and CH2 measures the voltage across Z.

Impedance of a Resistor

To verify the measurement method, a circuit was constructed with Z = R = 1 kΩ using two 1 kΩ resistors connected in series. Since the series resistor and the unknown component are equal, the expected amplitude of CH1 is twice that of CH2. Consequently, the MATH channel (CH1 − CH2), representing Vᵆ, should have the same amplitude as CH2. This was confirmed experimentally, validating the measurement approach.

Challenge: Measuring the Internal Resistance of the Signal Generator

In an ideal signal generator, the output resistance is zero; however, in practice a finite internal resistance r is always present. To determine r, two resistors with significantly different values were selected — 200 Ω and 9.74 kΩ (the latter verified by multimeter, as abrasion had shifted its nominal value of 10 kΩ) — in order to minimise the effect of measurement uncertainty on the result.

With R₁ = 200 Ω, the voltage across the resistor was measured as V₁ = 195 mV. With R₂ = 9.74 kΩ, the measured voltage was V₂ = 250 mV. Applying the potential divider equation for each case:

Vₛ · R₁ / (R₁ + r) = V₁        Vₛ · R₂ / (R₂ + r) = V₂

Dividing the two expressions to eliminate Vₛ and rearranging yields:

r · (R₂·V₁/V₂ − R₁) = (1 − V₁/V₂) · R₁R₂

Substituting the measured values gives r = 57.94 Ω. This non-negligible internal resistance confirms that the generator deviates from ideal behaviour and should be accounted for in precision measurements.